Optimization of Ethanol Production from Palmyra Sap by

Zymomonas mobilis using Response Surface Methodology (RSM)

RUTH CHRISNASARI1, AGUSTIN KRISNA WARDANI2, AND UNTUNG MURDIYATMO2

1Departement of Biology, Faculty of Biotechnology, University of Surabaya, Surabaya 60292, Indonesia

2Departement of Food Technology, Faculty of Agricultural Technology, Brawijaya University, Malang 65145, Indonesia

Statistical experimental design using a Box-Wilson central composite design (CCD) was used to optimize quantitative effects of sugar concentration, urea concentration, and inoculums concentration on ethanol production. It was found that palmyra sap could be used as suitable substrate for the ethanol production using Zymomonas mobilis (NRRL B-14234). A maximum ethanol concentration of 58.97 g/L was obtained after optimizing the parameters of fermentation.  The optimum values were sugar concentration 206,01 g/L, urea concentration 3,16 g/L, and inoculums concentration 23,05% v/v with ethanol yield was 0,3039. A high similarity was observed between the predicted and experimental results, which reflected the accuracy and applicability of RSM to optimize the process for ethanol production.

Key words: ethanol, palmyra sap, Zymomonas mobilis, Response Surface Methodology.

Ethanol is believed to be one of the best alternatives to replace gasoline because ethanol is a renewable source and environmentally friendly. Ethanol can be produced from several raw material, one of them is palmyra sap (Borassus flabellifer). Palmyra sap is sugar syrup from palmyra tree, known has more complete nutrition eg. sugar, protein, nitrogen, minerals, vitamine B complex that impelled the growth of microorganism (Morton, 1988). Palmyra sap is an agricultural product abundantly available in Indonesia, especially on Tuban and the northern coastal area of Java Island and is an alternative substrate for producing ethanol.

Zymomonas mobilis is a superior bacteria because can produce ethanol at more than double the reported rates for yeasts and has higher ethanol tolerant up to 13% (Jeffries, 2005; Seo et al., 2005). Recent process development has demonstrated the potential superiority of this organism in other aspects of industrial ethanol production.

To develop a process for maximum production of ethanol, standardization and optimization fermentation is crucial. Fermentation process of ethanol needs precise concentration of sugar as carbon source, urea as nitrogen source, and amount of inoculums.  Optimization by the classical method using a single dimensional search involving changing one variable while fixing the others at a certain level is laborious and time consuming, especially when the number of variables is large. These drawbacks of single factor optimization process can be eliminated by optimizing all the affecting parameters collectively by Central Composite Design using Response Surface Methodology (RSM).

Corresponding author:

Tel.: +6231 2981 399; Fax: +6231 2981 1278

E-mail addresses: ruth_c @ubaya.ac.id

Optimization using Response Surface Methodology (RSM) is the one suitable method for identifying the effect of individual variables and for seeking the optimum conditions for multivariable system efficiently. This method has been successfully applied to optimize ethanol fermentation and other fermentation media (Sunitha et al., 1998; Ambati and Ayyanna, 2001; Ratnam, 2001; Ratnam et al., 2003; Ratnam et al., 2005; Bandaru et al., 2006). A detailed account of this technique has been outlined (Cochran and Cox, 1968). Basically, this optimization process involves three major steps: performing the statistically designed experiments, estimating the coefficient in a mathematical model and predicting the response and checking the adequacy of the model. (Ratnam et al., 2005).

In this study, the RSM approach was adopted to locate optimum level of sugar concentration, urea concentration and inoculums concentration for ethanol fermentation from palmyra sap using Zymomonas mobilis, since these parameters play a key role in the enhancement of ethanol yield. Optimization was done with Box-Wilson RSM Central composite experimental design (CCD).

MATERIALS AND METHODS

Substrate. Palmyra sap is sugar syrup from palmyra tree (Borassus flabellifer), which is collected from Tuban, East Java, Indonesia.

Bacterial Strain. Zymomonas mobilis ZM4 (NRRL B-14234) obtained from ARS Culture Colection National Center for Agricultural Utilization Research, Preoria Il, USA, was used throughout the study.

Growth Medium and Growth Conditions. Z. mobilis was maintained on medium having composition (g/l): glucose, 100; yeast extract, 10; KH2PO4, 1; (NH4)2SO4, 1; MgSO4·7H2O, 0.5 and the cells were grown at a temperature of 35 ◦C and pH of 5.5.

Production Media and Fermentation. The fresh palmyra sap filtered and concentrated by boiling for ± 2-3 h until total sugar concentration ± 450g/L. Fermentation medium made from dilution of concentrated palmyra sap using several sugar concentration (115,9; 150, 200, 250 and 284,1 g/L). Added with several concentrations of urea (1,319; 2, 3, 4 and 4,682 g/L). Medium sterilized and added with several concentrations culture inoculums (11,59; 15, 20, 25 and 28,41 %v/v). Fermentation was carried out in batch condition using waterbath shaker. Fermentation condition was maintained at temperature 30 oC with initial pH 7 and incubated for 60 hours.

Analytical Methods. Reduction sugar was estimated using DNS method (Miller, 1959), total sugar (Ceirwyn, 1995) and total nitrogen using Mikro Kjeldahl method (Apriantono, 1989). Ethanol was estimated by spectroscopy method (megazyme etanol kit), cell concentration using optical density (OD) and cell amount with haemocytometer.

Experimental Design and Optimization. The aim of this study was to locate optimum level of sugar concentration, urea concentration and inoculums concentration for ethanol production from palmyra sap using Zymomonas mobilis.  Central composite experimental design (CCD, Box and Wilson 1951) was used in the optimization of ethanol production. Sugar concentration (X1, g/l), urea (X2, g/l), inoculums (X3, % v/v), were chosen as independent variables and ethanol concentration (Yi, g/L) was used as output variable. For statistical calculations the variables Xi were coded as Xi according to Equation (1).

xi = ,	i = 1,2,3,....k			(1)

Where, xi is the dimensionless value of an independent variable, Xi is the real value of an independent variable, xi; is the real value of the independent variable at the center point and ∆xj is step change. Table 1 shows Independent variable that used in this experimental plan.

Table1.Independent variable in the experimental plan

| Variable | Coded Level |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | -1,682 | -1 | 0 | 1 | 1,682 |
| Total Sugar (g/L), X1 | 115,9 | 150 | 200 | 250 | 284,1 |
| Urea (g/L), X2 | 1,318 | 2 | 3 | 4 | 4,682 |
| Inoculums (% v/v), X3 | 11,59 | 15 | 20 | 25 | 28,41 |

A 23-factorial CCD, with six axial points (α =√3) and six replications at the center points (n0 = 6) leading to a total number of 20 experiments was employed in Table 2.

The second degree polynomials (Equation (2)) were calculated with the statistical package (Stat-Ease Inc, Minneapolis, MN, USA) to estimate the response of the dependent variable:

Yi = b0 + b1X1 + b2X2 + b3X3 + b11+ b22+b33+

b12X1X2 + b23X2X3 + b13X1X3                           (2)

where Yi is the predicted response, X1, X2, X3 are independent variables, b0, is the offset term,  b1, b2, b3 are linear effects, b11,b22, b33 are squared effects and b12, b23, b13 are interaction terms.

Table2. The CCD matrix employed for three independent variables

| Experiment Number | X1 | X2 | X3 |
| --- | --- | --- | --- |
| 1 | -1 | -1 | -1 |
| 2 | 1 | -1 | 1 |
| 3 | -1 | 1 | 1 |
| 4 | 1 | 1 | -1 |
| 5 | 0 | 0 | 0 |
| 6 | 0 | 0 | 0 |
| 7 | -1 | -1 | 1 |
| 8 | 1 | -1 | -1 |
| 9 | -1 | 1 | -1 |
| 10 | 1 | 1 | 1 |
| 11 | 0 | 0 | 0 |
| 12 | 0 | 0 | 0 |
| 13 | -1,682 | 0 | 0 |
| 14 | 1,682 | 0 | 0 |
| 15 | 0 | -1,682 | 0 |
| 16 | 0 | 1,682 | 0 |
| 17 | 0 | 0 | -1,682 |
| 18 | 0 | 0 | 1,682 |
| 19 | 0 | 0 | 0 |
| 20 | 0 | 0 | 0 |

RESULTS

RSM is a sequential procedure with an initial objective of leading the experimenter rapidly and efficiently to the general vicinity of the optimum (Ratnam et al., 2005). Since the location of the optimum is unknown prior to running RSM experiments, it makes sense to have a design that provides equal precision of estimation in all directions is employed.

The three factors which influence highly the fermentative production are sugar concentration, urea concentration and inoculums concentration. Using CCD, a total number of 20 experiments with different combinations of sugar, urea, inoculums were performed (Tables 1 and 2). The response was taken at the maximum ethanol production which was observed at 60 h. The results were analyzed using the analysis of variance (ANOVA) and the estimation model analysis was done using Sequential Model Sum of Squares, Lack of Fit Tests and Model Summary Statistics. Prediction model that might be occur from response surface method were linear, 2FI (Two factor interaction), kuadratic and cubic. The result showed that suggestion model was Quadratic. The following second order polynomial equation was found to represent the ethanol production adequately:

Yi =	-234.38018	 + 1.90689X1 + 32.22936X2 + 3.72154X3 - 0.057275X1X2 + 0.015335X1X3 + 0.51575 X2X3 - 4.93427x10-3X12 - 5.27344 X2 - 0.18322X32                                      (3)

The coefficients of the regression model (Eq. (3)) calculated are listed in Table 3, in which they contain three linear, three quadratic, three interaction terms and one block term. The effects of all three parameters, i.e. sugar, urea and inoculums and their interactions with each other on percent ethanol concentration were found to be significant (P value (Prob>F) lower than 0.1000 indicate the model terms are significant).

The corresponding analysis of variance (ANOVA) also carried out to check the best fit of the model with F value was 1325.71. F value for lack of fit test was 0.69 means lack of fit of the model was not significant. Non-significant lack of fit is good. The coefficient of determination R2=0.9993 which implies that 99.93% of the sample variation in the ester yield is attributed to the independent variables. The R2 value also indicates that only 1% of the variation is not explained by the model. The value of R is 0.9986.

Table 3. ANOVA for full quadratic model

| Source | Sum of Squares |  | df | Mean Square |  | F value |  | P value Prob > F |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block | 4.59 |  | 2 | 2.30 |  |  |  |  |  |  |
| Model | 3303.77 |  | 9 | 367.09 |  | 1325.71 |  | < 0.0001 | Significant |  |
| A-Sugar | 158.16 |  | 1 | 158.16 |  | 571.19 |  | < 0.0001 | Significant |  |
| B-Urea | 4.15 |  | 1 | 4.15 |  | 14.99 |  | 0.0047 | Significant |  |
| C-Inoculums | 346.24 |  | 1 | 346.24 |  | 1250.42 |  | < 0.0001 | Significant |  |
| AB | 65.61 |  | 1 | 65.61 |  | 236.94 |  | < 0.0001 | Significant |  |
| AC | 117.58 |  | 1 | 117.58 |  | 424.64 |  | < 0.0001 | Significant |  |
| BC | 53.20 |  | 1 | 53.20 |  | 192.13 |  | < 0.0001 | Significant |  |
| A2 | 2191.23 |  | 1 | 2191.23 |  | 7913.53 |  | < 0.0001 | Significant |  |
| B2 | 400.45 |  | 1 | 400.45 |  | 1446.22 |  | < 0.0001 | Significant |  |
| C2 | 302.12 |  | 1 | 302.12 |  | 1091.10 |  | < 0.0001 | Significant |  |
| Residual | 2.22 |  | 8 | 0.28 |  |  |  |  |  |  |
| Lack of Fit | 1.18 |  | 5 | 0.24 |  | 0.69 |  | 0.6663 | Not significant |  |
| Pure error | 1.03 |  | 3 | 0.34 |  |  |  |  |  |  |
| Cor Total | 3310.58 |  | 19 |  |  |  |  |  |  |  |
| Std. Dev. |  | 0.53 |  |  |  |  | R Squared |  |  | 0.9993 |
| Mean |  | 42.77 |  |  |  |  | Adj R-Squared |  |  | 0.9986 |
| C.V. % |  | 1.23 |  |  |  |  | Pred R-Squared |  |  | 0.9944 |
| PRESS |  | 18.46 |  |  |  |  | Adeq Precision |  |  | 100.278 |

Table 4. Experimental and the predicted for ethanol production

| X1 | X2 | X3 | Ethanol (g/L) |  |
| --- | --- | --- | --- | --- |
| Sugar (g/L) | Urea (g/L) | Inoculums (% v/v) | Actual | Predicted |
| 150 | 2 | 15 | 31.92 | 31,39 |
| 250 | 2 | 15 | 36.11 | 36,26 |
| 150 | 4 | 15 | 30.16 | 30,86 |
| 250 | 4 | 15 | 24.48 | 24,27 |
| 150 | 2 | 25 | 28.43 | 28,64 |
| 250 | 2 | 25 | 49.54 | 48,84 |
| 150 | 4 | 25 | 38.57 | 38,42 |
| 250 | 4 | 25 | 46.64 | 47,16 |
| 115.91 | 3 | 20 | 17.45 | 17,30 |
| 284.09 | 3 | 20 | 28.62 | 28,75 |
| 200 | 1.32 | 20 | 43.42 | 43,93 |
| 200 | 4.68 | 20 | 42.60 | 42,08 |
| 200 | 3 | 11.59 | 36.57 | 36,49 |
| 200 | 3 | 28.41 | 53.37 | 53,43 |
| 200 | 3 | 20 | 58.10 | 57,92 |
| 200 | 3 | 20 | 57.92 | 57,92 |
| 200 | 3 | 20 | 57.01 | 57,92 |
| 200 | 3 | 20 | 58.40 | 57,92 |
| 200 | 3 | 20 | 58.20 | 57,92 |
| 200 | 3 | 20 | 57.89 | 57,92 |

DISCUSSION

The parity plot showed a satisfactory correlation between the values of experimental values and predictive values (Fig. 1), wherein, the points cluster around the diagonal line which indicates the good fit of the model, since the deviation between the experimental and predictive values was less.

Fig.1. Parity plot showing the distribution of experimental vs. predicted values of ethanol yield

Optimum level for sugar concentration, urea concentration and inoculums concentration could be predicted using the equation model. Figures 4–6 represent the iso-response contour and surface plots for the optimization of fermentation conditions of ethanol production.

Fig 2. Response surface and contour plot of sugar vs. urea concentration on ethanol production (inoculums was kept constant at 20% v/v/).

Fig 3. Response surface and contour plot of inoculums vs. sugar concentration on ethanol production (urea was kept constant at 3 g/L).

Fig 4. Response surface and contour plot of urea vs. inoculums on ethanol production (sugar concentration was kept constant at 200 g/L).

The effects of the sugar and urea concentration on the ethanol production showed in Fig. 2. An increase in the sugar with urea up to the optimum point increased the ethanol production to a maximum level and a further increase in the sugar with urea the trend is reversed. The optimum for maximum ethanol production lies near the centre point of the sugar and urea. The interaction effect of the inoculums and sugar concentration on the ethanol production in Fig. 3 clearly indicates a proper combination for production of ethanol. An increase in the inoculums with sugar increased the ethanol production gradually but at a higher inoculums and sugar the trend is reversed. A similar effect on the response was observed for the urea at any level of inoculums concentration an increase in the urea with inoculums up to the optimum point increased the ethanol production to maximum level and a further increase in the urea with inoculums decreased the ethanol production is shown in Fig. 4.

Therefore, an optimum was observed near the central value of sugar, urea and inoculums. The optimum conditions for maximum ethanol concentrations were obtained were sugar concentration 206,01 g/L, urea concentration 3,16 g/L, and inoculums concentration 23,05% v/v. A maximum ethanol concentration of 58.97 g/L was obtained at these optimum parameters. The experimental and predicted ethanol production at optimum conditions of fermentation were also determined (Table 5).

Table 5. 	The Optimum values of experimental and predicted yields for ethanol.

| Variables | Optimum Value | Optimum Ethanol Yield (g/L) |  |
| --- | --- | --- | --- |
|  |  | Experimental | Predicted |
| Sugar (g/L) | 206,01 | 58,97 | 59.77 |
| Urea (g/L) | 3,16 |  |  |
| Inoculums (%v/v) | 23,05 |  |  |

Based on result in Table 5, a high similarity was observed between the predicted and experimental results, which reflected the accuracy and applicability of RSM to optimize the process for ethanol production.

Yield ethanol production calculated from the experimental data:

RSM Optimization with CCD enables to find the importance of factors at different levels. A high similarity was observed between the predicted and experimental results, which reflected the accuracy and applicability of RSM to optimize the process for ethanol production. The results of this study have clearly indicated RSM is an effective method for maximum production of ethanol using palmyra sap Zymomonas mobilis.

The maximum ethanol concentration 58,97 g/L was obtained after optimizing the parameters of fermentation. The optimum values were sugar concentration 206,01  g/L, urea concentration 3,16  g/L, and inoculums concentration 23,05% v/v. The yield of ethanol production at optimum condition is 0,3039.

ACKNOWLEDGEMENTS

The authors thank to ASENDO (Indonesian Associations Spirits and Ethanol) for the financial support of the research and Head of Agroindustrial Biotechnology Double Degree Master Program Brawijaya University for the helpfully guidance and for the chemical analysis reagents.

REFERENCES

Ambati, P. & Ayyanna, C. 2001. Optimizing medium constituents and fermentation conditions for citric acid production from palmyra jaggery using response surface method. World Journal of Microbiology and Biotechnology 17, 331–335.

Bandaru, V.V.R., S.R. Somalanka, D.R. Mendu, N.R. Madicherla, A. Chityala. 2006. Optimization of fermentation conditions for the production of etanol from sago starch by co-immobilized amyloglucosidase and cells of Zymomonas mobilis using response surface methodology. Enzyme and Microbial Technology 38: 209–214

Cochran, N.G. & Cox, G.M. 1968 Experimental Designs. John Wiley and Sons Inc., p. 611.

Davis, T.A. and Johnson, D.V. 1987. Current Utilization and Further Development of the Palmyra Palm (Borassus flabellifer L.,Arecaceae) in Tamil Nadu State, India. Economic Botany, 41(2):47-266.

Jeffries, T.W. 2005. Etanol fermentation on the move. Nature Biotechnology 23: 40-41

Morton, J.F. 1988. Notes on Distribution, Propagation, and Products of Borassus Palms (Arecaceae). Economic Botany (1988) 42(3):420-441

Ratnam, B.V.V, S. Subba Rao, M. Damodar Rao, M. Narasimha Rao, C. Ayyanna. 2005. Optimization of medium constituents and fermentation conditions for the production of ethanol from palmyra jaggery using response surface methodology. World Journal of Microbiology & Biotechnology (2005) 21:399–404

Seo, J.S, Chong H., Park HS, Yoon KO, Jung C, Kim JJ, Hong JH, Kim H, Kim JH, Kil JI, Park CJ, Oh HM, Lee JS, Jin SJ, Um HW, Lee HJ, Oh SJ, Kim JY, Kang HL, Lee SY, Lee KJ, Kang HS. 2004. The genome sequence of the etanologenic bacterium Zymomonas mobilis ZM4. Nature Biotechnology 23: 63-68

Sunitha, I., Subba Rao, M.V. & Ayyanna, C. 1998. Optimization of medium constituents and fermentation conditions for the production of L-glutamic acid by the coimmobilized whole cells of Micrococus glutanicus and Pseudomonas reptilivora. Bioprocess Engineering 18 , 353–359.